COMPUTER SCIENCE EPARTMENT

| Total Marks: | 04 |
| --- | --- |
| Obtained Marks: |  |

Data Communication and Computer Networks

Assignment#04

Submitted To: Mr. Fakhar

StudentName:	Mushahid Hussain	 RegNumber:	 2212408

COMPUTER SCIENCE DEPARTMENT

Instructions:

- Copied or shown assignments will be marked zero.

- Late submissions will result in marks deduction.

- Hand written assignments are not acceptable.

- If you have any query ,you can e-mail it on: fakhar.islam@szabist-isb.edu.pk

Question: Explain and draw Digital data to Analog signal encoding schemes such as ASK,FSK and PSK.

Digital Data to Analog Signal Encoding Techniques

Introduction

In data communication systems, digital data often needs to be transmitted over analog transmission media such as telephone lines or wireless channels. To achieve this, digital data is converted into analog signals using a process called modulation. The analog signal is generated by modifying a high-frequency carrier wave according to the digital input data.

The main digital data to analog signal encoding techniques are:

- Amplitude Shift Keying (ASK)

- Frequency Shift Keying (FSK)

- Phase Shift Keying (PSK)

1. Amplitude Shift Keying (ASK)

Explanation

Amplitude Shift Keying is a modulation technique in which the amplitude of the carrier signal is varied in accordance with the digital data, while the frequency and phase remain constant.

In Binary Amplitude Shift Keying (BASK):

- Binary 1 is represented by a carrier wave with higher amplitude

- Binary 0 is represented by a carrier wave with lower or zero amplitude

Diagram

Binary Data:   1      0      1      1      0

ASK Signal:

~~~~~~        ~~~~~~  ~~~~~~

Advantages

- Simple to implement

- Requires less bandwidth

Disadvantages

- Highly susceptible to noise

- Poor performance in noisy communication channels

2. Frequency Shift Keying (FSK)

Explanation

Frequency Shift Keying is a modulation technique in which the frequency of the carrier signal is changed according to the digital data, while the amplitude and phase remain constant.

In Binary Frequency Shift Keying (BFSK):

- Binary 1 is transmitted using a higher frequency (f₁)

- Binary 0 is transmitted using a lower frequency (f₀)

Diagram

Binary Data:   1        0        1        0

FSK Signal:

~~~~~~~~   ~~~~   ~~~~~~~~   ~~~~

Advantages

- Better noise immunity compared to ASK

- Suitable for wireless communication

Disadvantages

- Requires more bandwidth

- More complex than ASK

3. Phase Shift Keying (PSK)

Explanation

Phase Shift Keying is a modulation technique in which the phase of the carrier signal is varied according to the digital data, while the amplitude and frequency remain constant.

In Binary Phase Shift Keying (BPSK):

- Binary 1 is represented by a carrier wave with 0° phase

- Binary 0 is represented by a carrier wave with 180° phase shift​

Diagram

Binary Data:   1        0        1        0

PSK Signal:

~~~~~~~~  ~~~~~~~~  ~~~~~~~~  ~~~~~~~~

0°        180°       0°        180°

Advantages

- High resistance to noise

- Efficient use of bandwidth

- Widely used in modern communication systems

Disadvantages

- Complex receiver design

- Requires accurate phase synchronization

Comparison of Encoding Techniques

| Technique | Parameter Varied | Noise Resistance | Bandwidth Requirement |
| --- | --- | --- | --- |
| ASK | Amplitude | Low | Low |
| FSK | Frequency | Medium | Medium |
| PSK | Phase | High | Efficient |

Conclusion

Digital data to analog signal encoding is essential for transmitting digital information over analog communication channels. ASK is simple but highly noise-sensitive, FSK offers better noise performance at the cost of increased bandwidth, and PSK provides the best noise immunity and efficiency, making it suitable for modern high-speed communication systems.