| Total Marks: | 04 |
| --- | --- |
| Obtained Marks: |  |

Computer Networks and Data Communication

Assignment # 01 Section-B

Last date of Submission: Oct 6th, 2025

Submitted To:	Mr. Fakhar

_______________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Student Name:	Ubaid Bin Waris

_______________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Reg Number:	2212416

_______________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Instructions:

- Copied or shown assignments will be marked zero.

- Late submissions will result in marks deduction.

- Hand written assignments are not acceptable.

- If you have any query ,you can e-mail it on: fakhar.islam@szabist-isb.edu.pk

Question 1 Differentiate between standard and protocols. Why standards are important for protocols? (CLO 1)

### Difference Between Standards and Protocols

| Aspect | Standards | Protocols |
| --- | --- | --- |
| Definition | Standards are agreed-upon rules or guidelines that ensure uniformity and compatibility across systems and devices. | Protocols are specific sets of rules that define how data is transmitted and received over a network. |
| Purpose | To ensure that hardware and software from different manufacturers can work together smoothly. | To define communication procedures between devices or systems on a network. |
| Scope | Broader — covers formats, interfaces, voltages, frequencies, and communication rules. | Narrower — focuses mainly on data exchange and communication behavior. |
| Examples | IEEE 802.3 (Ethernet standard), ISO/OSI Model, ITU standards. | TCP/IP, HTTP, FTP, SMTP, DNS. |
| Defined By | Organizations such as IEEE, ISO, ITU, IETF. | Often developed and implemented based on network standards. |
| Nature | Standards provide the foundation for building and maintaining interoperability. | Protocols are implemented to achieve communication following those standards. |

### Why Standards Are Important for Protocols

Standards are essential for protocols because they:

- Ensure Interoperability: Allow devices and software from different vendors to communicate seamlessly.

- Provide Uniform Guidelines: Help in designing and implementing protocols consistently across systems.

- Enhance Reliability: Standardized protocols ensure stable and predictable network behavior.

- Support Global Communication: Without standards, communication systems would be incompatible across countries or networks.

- Facilitate Innovation: Developers can build new technologies that work with existing systems without compatibility issues.

Question 2  What are the 11 major tasks for a data communication system? (CLO 1)

### 11 Major Tasks for a Data Communication System

A data communication system performs several important tasks to ensure efficient, reliable, and accurate data transfer between devices.
The 11 major tasks are:

- Transmission System Utilization
Efficiently using the available communication medium (like cable or wireless) to transfer data.

- Addressing
Assigning unique addresses to devices or nodes so that data reaches the correct destination.

- Interfacing
Defining how devices (like computers, routers, or modems) connect and interact with the transmission medium.

- Signal Generation
Converting data into signals (analog or digital) that can be transmitted through the medium.

- Synchronization
Ensuring the sender and receiver are in sync so data is transmitted and received correctly.

- Exchange Management
Controlling how communication is initiated, maintained, and terminated between devices.

- Error Detection and Correction
Identifying and fixing errors that occur during data transmission to ensure data accuracy.

- Flow Control
Managing the rate of data transmission to prevent a fast sender from overwhelming a slow receiver.

- Recovery
Restoring communication and retransmitting lost or corrupted data in case of failure.

- Message Formatting
Structuring data in a predefined format or frame so that both sender and receiver interpret it correctly.

- Security and Privacy
Protecting data from unauthorized access, modification, or loss during transmission.

Question 3  Explain simplified data communication model with diagram. Also, explain data flow types in communication with examples. (CLO 1)

### Simplified Data Communication Model

A data communication model explains how data is transmitted between two devices. It shows the process of sending and receiving information over a communication medium. The basic elements of this model are:

- Sender (Source): The device or person that generates the data to be transmitted.
Example: A computer sending an email.

- Message: The information or data that needs to be communicated.
Example: Text, images, or audio.

- Transmitter (Encoder): Converts the message into signals suitable for transmission.
Example: A modem that changes digital data into signals.

- Transmission Medium (Channel): The physical path through which the signal travels.
Example: Copper wire, optical fiber, or radio waves.

- Receiver (Decoder): Converts the received signals back into the original form of data.
Example: A modem that converts signals back to digital data.

- Destination: The device or person that receives the message.
Example: The recipient’s computer.

- Noise: Any unwanted interference that may distort or alter the transmitted data.

In simple terms, the sender transmits data through a medium to the receiver, where the data is decoded and delivered to the destination. Noise can sometimes affect the accuracy of the transmitted data.

### Data Flow Types in Communication

Data flow refers to the direction in which data moves between two devices in a communication system. There are three main types of data flow:

- Simplex:
Data travels in only one direction, from sender to receiver. The receiver cannot send data back.
Example: Keyboard to CPU or CPU to monitor.

- Half Duplex:
Data travels in both directions, but only one direction at a time. The devices take turns in sending and receiving data.
Example: Walkie-talkie communication.

- Full Duplex:
Data travels in both directions simultaneously. Both devices can send and receive data at the same time.
Example: Telephone communication or modern computer networks.