Mid paper

You are tasked with troubleshooting connectivity in a small
network with two routers (R1, R2),
each connected to a PC (PC1 and PC2, respectively).

- Step
1: Create a topology with two routers connected via a serial interface.

- Step
2: Connect PC1 to R1’s LAN and PC2 to R2’s LAN.

- Step
3: Assign IP addresses:

- Step
4: Configure static routing on both routers for end-to-end communication.

- Step
5: Use the ping command from PC1 to PC2. Observe ICMP replies and analyze
what happens if a route is missing.

Deliverables: Screenshot of the ping test, static routing commands, and
an explanation of how ICMP helps detect network issues.

R1 Config

enable

configure terminal

interface GigabitEthernet0/0/0

ip address 192.168.0.254 255.255.255.0

no shutdown

interface Serial0/1/0

ip address 10.0.0.1 255.255.255.252

no shutdown

ip route 192.168.100.0 255.255.255.0 10.0.0.2

R2 config

enable

configure terminal

interface GigabitEthernet0/0/0

ip address 192.168.100.254 255.255.255.0

no shutdown

interface Serial0/1/0

ip address 10.0.0.2 255.255.255.252

no shutdown

ip route 192.168.0.0 255.255.255.0 10.0.0.1

A basic topology diagram is provided

showing a router, a switch, PCs, and Laptops.




Tasks:


A.   
Configure DHCP on the router so that all PCs receive
IP addresses automatically.


B.    
Verify successful IP assignment on each PC and test
connectivity between them using ping.


Include screenshots of
the DHCP pool, IP configurations, and ping results in your report.

enable

configure terminal

interface GigabitEthernet0/0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

ip dhcp pool LAN_POOL

network 192.168.1.0 255.255.255.0

default-router 192.168.1.1

dns-server 8.8.8.8

ip dhcp excluded-address 192.168.1.1

R1> enable

R1# configure terminal

! PC1 network

R1(config)# interface Gig0/0/0

R1(config-if)# ip address 10.0.0.1 255.0.0.0

R1(config-if)# no shutdown

! Link to R2

R1(config)# interface Serial0/0

R1(config-if)# ip address 192.168.12.1 255.255.255.0

R1(config-if)# no shutdown

R1(config)# router rip

R1(config-router)# version 2

R1(config-router)# no auto-summary

R1(config-router)# network 10.0.0.0

R1(config-router)# network 20.0.0.0

R1(config-router)# network 192.168.12.0

R1> enable

R1# configure terminal

! PC1 network

R1(config)# interface Gig0/0/0

R1(config-if)# ip address 10.0.0.1 255.0.0.0

R1(config-if)# no shutdown

! PC2 network

R1(config)# interface Gig0/0/1

R1(config-if)# ip address 20.0.0.1 255.0.0.0

R1(config-if)# no shutdown

! Link to R2

R1(config)# interface Serial0/0

R1(config-if)# ip address 192.168.12.1 255.255.255.0

R1(config-if)# no shutdown

R2(config)# router rip

R2(config-router)# version 2

R2(config-router)# no auto-summary

R2(config-router)# network 30.0.0.0

R2(config-router)# network 192.168.12.0

R1# show ip interface brief

R2# show ip interface brief

R1# do show ip route

R2# do show ip route

You need 1 switch and 4 PCs (simplest approach):

- PC1 and PC2 → Floor 1 → VLAN 11 → 192.168.11.0/24

- PC3 and PC4 → Floor 2 → VLAN 12 → 192.168.12.0/24

Configure VLANs on the Switch

Enter VLAN configuration

Switch> enable

Switch# configure terminal

Switch(config)# vlan 11

Switch(config-vlan)# name Floor1

Switch(config-vlan)# exit

Switch(config)# vlan 12

Switch(config-vlan)# name Floor2

Switch(config-vlan)# exit

Switch(config)# interface range fa0/1 - 2

Switch(config-if-range)# switchport mode access

Switch(config-if-range)# switchport access vlan 11

Switch(config-if-range)# exit

Switch(config)# interface range fa0/3 - 4

Switch(config-if-range)# switchport mode access

Switch(config-if-range)# switchport access vlan 12

Switch(config-if-range)# exit

Step 5: Assign IP Addresses to PCs

- PC1 → VLAN 11

  - IP: 192.168.11.10

  - Subnet mask: 255.255.255.0

- PC2 → VLAN 11

  - IP: 192.168.11.11

  - Subnet mask: 255.255.255.0

- PC3 → VLAN 12

  - IP: 192.168.12.10

  - Subnet mask: 255.255.255.0

- PC4 → VLAN 12

  - IP: 192.168.12.11

  - Subnet mask: 255.255.255.0

Note: No default gateway is needed if VLANs are not routing between each other.